AtlasPM · Embedded SoC · Key deliverable template

# EPD-D8 — Signoff-ready database handoff

Produced by EPD-10 Hierarchical Block Closure and Top-Level Assembly (Physical Design). Fill in each section, attach the completed document to this deliverable's Handover in AtlasPM, and date the handover when it is accepted.

## 1. Document control

| Field | Entry |
| --- | --- |
| Programme |  |
| Deliverable | EPD-D8 — Signoff-ready database handoff |
| Stage | Physical Design |
| Producing activity | EPD-10 — Hierarchical Block Closure and Top-Level Assembly |
| Owner | Hierarchical closure lead |
| Version / date |  |
| Status | Draft / For review / Signed |

## 2. Decision

☐ Signed off     ☐ Signed off with conditions     ☐ Not signed off

| Field | Entry |
| --- | --- |
| Summary |  |
| Conditions |  |
| Next review |  |

## 3. Baseline signed off against

| Item | Version / tag / ID |
| --- | --- |
| FFN tag the database is built from |  |
| Database version and handoff directory |  |
| SDC and UPF versions |  |
| PDK, library and signoff deck versions |  |

## 4. Entry criteria

| Criterion | Evidence (link or file) | Met |
| --- | --- | --- |
| Partitions defined in EPD-02 |  | ☐ |
| Interface budgets available from EARCH-09 |  | ☐ |
| Blocks converging in EPD-05 |  | ☐ |

## 5. Checks and results

| Check | Target / limit | Result | Status |
| --- | --- | --- | --- |
| Setup and hold timing across all corners and modes | WNS ≥ 0, TNS reported |  | ☐ |
| Block models correlated against flat analysis | Within the agreed tolerance |  | ☐ |
| DRC, LVS and antenna | Clean or waived |  | ☐ |
| Static and dynamic IR drop | Within budget |  | ☐ |
| Electromigration — power and signal | Clean at the hot corner |  | ☐ |
| Scan chains verified after reordering | All chains pass |  | ☐ |
| Fill and density | Compliant |  | ☐ |
| eMRAM, PMU and oscillator macro placement against vendor rules | Compliant |  | ☐ |
| ECO log reconciled with the netlist | Reconciled |  | ☐ |

## 6. Exit criteria

| Criterion | Evidence (link or file) | Met |
| --- | --- | --- |
| Block models correlated against flat analysis |  | ☐ |
| Every block closed to one shared criterion |  | ☐ |
| Top level assembled and verified before the final turn ends |  | ☐ |

## 7. Handoff file list

Every file signoff and tapeout will read, with its version and checksum.

| File (GDS/OASIS, netlist, SPEF, SDF, UPF, …) | Version | Checksum |
| --- | --- | --- |

## 8. Open issues

| ID | Description | Severity | Owner | Due | Disposition |
| --- | --- | --- | --- | --- | --- |

## 9. Waivers

| ID | Check / rule | Justification and risk | Condition / expiry | Approver |
| --- | --- | --- | --- | --- |

## 10. Known failure modes — confirm each is addressed

☐ Block models disagreeing with flat analysis. The top level is then closed against an abstraction that does not represent the block.

☐ Interface budgets set late. Blocks close internally and fail at their boundaries, and the fix requires reopening blocks that were declared done.

☐ Tile abutment not planned. Tiles connect by abutment rather than by top-level routing, and a pin or rail mismatch at the tile edge repeats across the whole array.

☐ Blocks closed to different criteria. Without an enforced standard, each block closes to what its owner considered sufficient, and the top level inherits the weakest.

☐ Top-level assembly left to the end. Assembly finds interface problems, and finding them after the final turn leaves no time to fix them.

## 11. Handover

Who receives this deliverable, and what they take from it.

| Receiving activity | What it takes | Received (date) |
| --- | --- | --- |
| ESO-03 — Multi-Corner Multi-Mode Signoff STA and Closure |  |  |
| ESO-04 — Full-Chip DRC, LVS, Antenna, and Density Verification |  |  |
| ESO-09 — Final Formal Equivalence and LVS Netlist Consistency |  |  |
| ETO-01 — Final GDSII / OASIS Assembly and Layer Map Verification |  |  |

## 12. Sign-off

| Role | Name | Decision | Signature | Date |
| --- | --- | --- | --- | --- |
| Hierarchical closure lead |  |  |  |  |
| Block physical owners |  |  |  |  |
| Top-level integration engineer |  |  |  |  |
| Timing engineer |  |  |  |  |
| Physical verification engineer |  |  |  |  |

## Revision history

| Version | Date | Change | Author |
| --- | --- | --- | --- |
| 0.1 |  | Template issued |  |